PERFORMANCE OF POLYNOMIALS AND ARTIFICIAL NEURAL NETWORKS

FOR MODELING GEOID SURFACE IN LAGOS STATE

Andrew Tolu Taiwo

(Matric Number: 209045042)

A Paper

SVY 904: Advanced Geometric Geodesy

Department of Surveying and Geoinformatics

University of Lagos

INTRODUCTION

Two height systems are used for mapping, navigation and engineering developments – orthometric height and ellipsoidal height. Orthometric height references the geoid or equipotential surface, which almost coincide with the mean sea level, while ellipsoidal height references the surface of an ellipsoid, the World Geodetic System 1984 (WGS 84). Orthometric height is the classical height system to which mapping and engineering activities are based. It is tied to a physical phenomenon, the mean sea level. But ellipsoidal height is not based on any physical feature, and therefore not reliable. However, Global Positioning System (GPS) produces ellipsoidal height. Since the use of GPS is pervasive in modern mapping, navigation, engineering developments and other geospatial activities, there is need to convert ellipsoidal heights to orthometric heights. This conversion can be achieved by adding geoid heights to ellipsoidal heights.

Moreover, the ellipsoid is easily modelled through GPS measurements at any point on the earth surface. The geoid is not easily modelled. This is because height differences based on mean seal level (or the geoid surface) need to be extended from points of known orthometric heights through spirit levelling, a task which is very tedious, time consuming, labour intensive, expensive, and difficult to carry through inaccessible remote areas.

In order to overcome this problem, researchers have proposed various polynomials which model local geoidal surface, and through which orthometric heights at some known points can be interpolated to other known points. Different polynomials best model different local regions with particular uncertainty (Nwilo et al., 2009; Odumosu et al., 2016; Erol, 2011, Oluyori et al., 2020). The development of local geoid surfaces aims basically at providing GPS users with an optimal transformation model between ellipsoidal heights and orthometric heights with respect to a given levelling datum. For a general discussion regarding theoretical and practical aspects of this problem, see Featherstone (1998).

These surfaces are also called geometric geoid surfaces (Eteje et al., 2018). They are mathematical interpolation surfaces fitted to existing geoid heights to enable geoid heights of new points to be determined using variables such as geographic or rectangular coordinates of the points. These surfaces include plane surface, bi-linear surface, second degree polynomial, third degree polynomial and fifth degree polynomial. The surface to be adopted as well as the degree and order of the polynomial depends on the size of the study area and the variation of the geoid heights. For a small area, the plane surface is used, for a relatively large area, the second and third order polynomial surfaces are used.

Nwilo et al., (2009) carried out geoid modelling technique based on geometrical interpolation approach. A one-degree polynomial of type equation (2) model was formulated using orthometric and ellipsoidal heights. From the analysis, it was observed that the use of the lower order polynomial to model the geoid surface gave the mean square errors of 0.060m.

Olaleye et. al., (2013) adopted the method of ‘satlevel’, in which the ellipsoidal height is used along with Orthometric height to model the geoid. They developed a one-degree, four-term polynomial model using least squares method from the combination of Orthometric and ellipsoidal heights along with space rectangular coordinates. They discovered through statistical tests and goodness of fit analysis that there is no significant difference between the values obtained with the derived model and observed values.

Odumosu et al., (2016) developed an empirical geoid model for Lagos State based on gravimetric and GPS levelling data. They calculated geoid heights by two methods – gravimetric geoid computation by evaluating Stoke’s integral and a one-degree polynomial of type equation (2). They compared the two results and got root mean square errors of 0.090m.

Recently, artificial neural networks models are employed in modeling local geoid surfaces and found to give better accuracies (Gullu et al., 2011, Akari et al., 2020, Cakir et al., 2014). Several soft computing methods have been applied as alternative techniques to produce accurate results in the prediction of geoid undulation, including the artificial neural network (ANN), least square support vector machines (LS-SVM), and adaptive neuro-fuzzy inference systems (ANFIS). Among these methods, the ANN has been found to produce reliable results (Konakoglu and Akar, 2021). Several researches have studied the performance of ANN with a different network topology in the modeling of a local geoid surface (Lin, 2007; Akcin and Celik, 2013; Erol and Erol, 2013; Albayrak et al., 2020).

The global geoid models such as Earth Geopotential Model 2008 (EGM08) make geoid height available globally. Since they are not regional or local, the global geoid models have a disadvantage that the geoid heights they provide are not reliable for mapping and engineering development locally. EGM08 global geoid model provides undulation values N with an uncertainty of ±37cm and ±22cm respectively (Gomaa D., et al, 2010). Okiwelu (2011) found them to be between 16cm and 25cm in Nigeria. This is a good reason for creating local and regional geoid models.

Geoid Modeling

Polynomial of degree n that can model the local geoid surface can be derived from equation (1) (Kirici and Sisiman, 2017) provided the coordinates, ellipsoidal heights and orthometric heights of enough points in the area are available.

Nx,y= i=0mj= k-1, i=0kaij.xiyj                                                                                                     (1)

tber of re  of degree n can be derived from equation ares are l geoid surface. d out odelled through G and geodial where xi and yj represent the position coordinates, aij symbolizes the polynomial coefficients, and m is the degree of the polynomial.

The polynomial of smallest degree used in literature is the one-degree with three parameters, also called plane surface, which is expressed in equation (2) (Nwilo et al., 2009; Odumosu et al., 2016).

N = a00 +  a10X +  a01Y									(2)

Sanlioglu et al. (2009) gave a second degree polynomial, or multi-quadratic model, as in equation (3):

N= a00+ a10x+a01y + a11xy+ a20x2+a02y2+ a21x2y+ a12xy2+ a22x2y2														(3)

Kirici and Sisman (2017) studied a third degree polynomial expressed in equation (4):

N= a00+ a10x+a01y + a11xy+ a20x2+a02y2+ a21x2y+ a12xy2+a30x3+a03y3 													(4)

Awange et al (2010) and Eteje et al., (2019) worked on a fifth-degree polynomial geoid surface, also called quintic polynomial, expressed in equation (5).

N= a00+ a10x+a01y + a11xy+ a20x2+a02y2+ a21x2y+ a12xy2+a30x3+a03y3 + a22x2y2+a31x3y+a13xy3+a40x4+a04y4+a32x3y2+a23x2y3+a41x4y+a14xy4+a50x5+a05y5										(5)

It is noted that the position coordinates x and y are the observables while the polynomial coefficients (a0,  a1,… au-1) are the unknown parameters in the polynomial models and are determined according to least squares adjustment method (Eteje and Oduyebo, 2018). The summation of observation (Ni) and its correction (residual, vi) at each station is described with a linear function of unknowns given by Ayeni (2011) and Ghilani (2018) as equation (6):

La = AXa-V										(6)

where La is the adjusted observations, A is the matrix of coefficients, Xa is the vector of adjusted parameters, and V  is the vector of residuals.

According to Kotsakis and Sideris (1999), one way to solve equation (6) for geoid surface modelling is to neglect the residuals V. This means that the surface will be modelled by pre-determined parametric equations such as the polynomials above, which relate the observations to the unknown parameters only. This is expressed by equation (7), the solution of which is given by equation (8).

La = AXa											(7)

Xa=ATA-1ATL										(8)

where ATA is the normal matrix, and ATL is the matrix of numeric terms. Comprehensive way of computing equation (8) for geoid surface models can be found in Eteje and Oduyebo, (2018).

Residuals, that is, the differences between observed and computed geoidal heights, are computed from equation (9):

V=La-L0											(9)

where  L0 is the observed geoidal heights and La is the computed geoidal height, which must be computed from the local geoid model achieved after substituting the parameters Xa and the position coordinates x and y into equation (7).  It should be noted that the coefficient matrix A collects the coefficients of the parametric equations together so that the computed geoidal heights is obtained from equation (7); that is, by simply multiplying the two matrices A and Xa. Thus, geoidal heights of new known points can be computed (or interpolated) by the same method described here.

Artificial Neural Networks (ANN)

Artificial Neural Network (ANN) and its various forms are commonly used in machine learning data processing and analysis because of their ability to learn from both linear and non-linear data.   Two ANNs, Multilayer perceptron (MLP) and Generalized Regression Neural Network (GRNN), are discussed and implemented to predict geoid heights in this study. MLP is a common feedforward artificial neural network that generates a set of outputs from a set of inputs. An MLP is characterized by several layers of input nodes connected as a directed graph between the input and output layers. MLP is a deep learning method that uses back-propagation algorithm for training the network.

MLPs are popular among existing neural networks due to their simple structures and ease of mathematical analysis. The input layer consists of the inputs to the network, followed by a hidden layer consisting of neurons or hidden units, and then an output layer representing classes or patterns. The input pattern is presented to the network via the input layer, and the input signals are passed to the nodes in the next layer in a feed-forward manner. The summation of the output is called the output layer. MLPs, as shown in Figure 1, are controlled by setting and adjusting weights between nodes. Initial weights are usually set at some random numbers and then they are adjusted during training.

Input Layer

Layer 1

Hidden Layer

Layer 2

x2

x3

x1

a3(2)

a2(2)

a1(2)

hw(x)

Output Layer

Layer 3

Figure 1: A 3 by 3 by 1 multilayer feedforward neural network architecture. (Source: Taiwo, 2023)

GRNN, as suggested by Specht (1991), is a feed forward ANN model consisting of four layers: input layer, pattern layer, summation layer and output layer. Unlike backpropagation ANNs, iterative training is not required. Each layer in the structure consists of different numbers of neurons and the layers are connected to the next layer in turn. (Specht, 1993). The GRNN has the advantages of fast training, simple structure, and global convergence (Yuan et al., 2019). The main parameter in the GRNN training process is the spread parameter (σ), which directly affects the prediction accuracy. This parameter is often chosen by trial-and-error experiments (Haidar et al., 2011).

Figure 2: The general structure of GRNN (Source: https://github.com/muhendis/Generalized-regression-neural-networks-library-from-scratch?tab=readme-ov-file )

GRNN makes use of both training and testing datasets, each of which contain input-output mapping. GRNN trains itself with the training dataset in order to learn the data. By feeding GRNN with the test dataset, it gives the output or predicts the result, and then estimates the output of the test dataset using weighted average of the outputs of the training dataset. The weight is calculated using Euclidean distance between the training dataset and test dataset. If the distance is large then the weight will be less, and if the distance is small it will give more weight to the output.

Equation (10) shows the regression of the dependent variable y with respect to the independent variable x, if the joint probability density function f (x, y) is known.

E[y|X]=-∞∞yf(X,y)dy-∞∞y(X,y)dy									(10)

Solving equation (10) gives equation (11):

YX=Yie-(di2/2σ2)e-(di2/2σ2)										(11)

where, Yi is the output of the input sample i.  x is the input sample and xi is the training sample. di2=x-xiTx-xi is the Euclidean distance from the x  and xi. It signifies how much the training sample can contribute to the output of the test sample.  If  di2  has small value then it will contribute more value to the output but if it is a big value that means it will contribute less to the output.

The term e-(di2/2σ2) is the activation function. It decides how much weight the training sample will contribute. If di2  is a small value, the term e-(di2/2σ2) returns a relatively large value. If di2 is a large value, the term e-(di2/2σ2) returns a relatively small value. If di2 is zero, then e-(di2/2σ2))  returns a value of test data that is equal to the value of training data.

It should be noticed that equation (11) has only one unknown parameter σ, called spread constant, which can be tuned by training process to an optimum value so that the error will be very small.

This study aims to model geoid surface in Lagos State Nigeria with polynomial models and artificial neural networks, and compare their accuracies. Performance of these models to optimally represent the geoid in the study area will be investigated. The results shall be compared with global geoid model EGM08, and the enrichment of the global geoid (that is, reducing it to fit the local geoid surface) shall be attempted. Thus, the difference between the zero reference surface for the determination of the orthometric heights in Lagos State in relation to the zero geoid surface of the EGM08 shall be determined.

MATERIALS AND METHODOLOGY

Data

Two datasets were used in this study. The first dataset was sourced from the Office of the State Surveyor General of Lagos State. The dataset was derived from comprehensive mapping of the state carried out in 2010. It comprises coordinates, orthometric heights and ellipsoidal heights of 1,165 control points (first-order, second-order and third-order) distributed across the state.

The second dataset is the geoid heights of those control points derived from EGM08 model. It was downloaded from University NAVSTAR Consortium (UNAVCO) website at https://www.unavco.org/software/geodetic-utilities/geoid-height-calculator/geoid-height-calculator.html.

Table 1: Sample dataset

| CONTROL ID | EASTING | NORTHING | ORTHOMETRIC HEIGHT | ELLIPSOIDAL HEIGHT | GEOID HEIGHT | ORDER |
| --- | --- | --- | --- | --- | --- | --- |
| XST101 | 535822.2 | 712062.9 | 3.608 | 26.063 | 22.455 | FIRST |
| XST102 | 533798 | 715890.8 | 3.424 | 26.089 | 22.666 | FIRST |
| XST103 | 530971.6 | 724703.5 | 18.757 | 41.872 | 23.115 | FIRST |
| XST104 | 529447.6 | 728733.2 | 32.178 | 55.482 | 23.303 | FIRST |
| YTT28-1 | 512745.4 | 718221.4 | 5.329 | 28.303 | 22.974 | SECOND |
| YTT28-10 | 518388.5 | 714448.9 | 6.812 | 29.537 | 22.725 | SECOND |
| YTT28-100 | 531140.5 | 731154.4 | 13.849 | 37.239 | 23.389 | SECOND |
| YTT28-101 | 530376.1 | 730032.7 | 40.124 | 63.474 | 23.35 | SECOND |
| ZTT-36-36 | 568467.2 | 710434.2 | 4.692 | 26.928 | 22.236 | THIRD |
| ZTT14-1A | 603337.6 | 731753.4 | 5.941 | 29.027 | 23.086 | THIRD |
| ZTT2-57A | 586045.2 | 711697.1 | 4.61 | 26.884 | 22.273 | THIRD |

Performance Metrics

In assessing the performance of geometric surface model in a particular region, several tests can be applied to the results of the least-squares adjustment. Fotopoulos (2003), Lambrou and Pantazis (2005), and Alevizakolu and Lambrou (2011) suggested the following:

- root mean square error (RMSE),

- goodness of fit,

- cross-validation,

- testing of parameter significance, and

- testing for the necessity of a surface of higher degree

Before the new model can be reliably used for interpolation or computation of geoidal heights of new stations, its reliability is measured with the classic root mean square error (RMSE). This is done by finding the square root of the mean sum of squares of the residuals by using equation (12):

RMSE=VTVn										(12)

where n is the number of observation equations.

The type of surface model used in the least-squares adjustment affects the estimated values of the RMSE. The model that results in the smallest set of residuals is deemed to be the most appropriate ('best' fit). Researchers have discovered that this method is not reliable as the only means for selecting between different models because the lowest RMSE usually corresponds to the highest order model. In fact, as the number of parameters in the model increases, the associated RMSE decreases (Fotopoulos, 2003). Therefore, this method is valid for testing the precision of the model, but it should not be interpreted as the accuracy or the prediction capability of the model. Thus, the same models implemented for the same data should be subjected to statistical testing methods such as measure of goodness of fit.

A statistical measure of the goodness of model fit for a discrete set of points is given by the coefficient of determination, denoted by R2. It can be described as the ratio of the sum of the squares due to the fit, to the sum of the squares about the mean of the observations. Thus, the coefficient of determination varies between 0 and 1 (0 ≤ R2 ≤ 1) and the closer the value is to one, the smaller the residuals and hence the better the fit.

Parameter significance is tested on each parameter in order to assess whether they are statistically important at a certain confidence level using the formula in equation (13):

σai . z95%≤ai											(13)

where σai is the standard deviation of each unknown parameter ai, and z95% is the coefficient of the normal distribution for one dimension arrays for confidence level 95%. If equation (13) is valid, then the parameter ai will be considered as being statistically important.

The necessity of using a surface of higher degree in geometric geoid modelling can be determined by using the relationship in equation (14):

r1σ1r2σ2≤F1r2											(14) where r1, r2 are degrees of freedom of the smaller degree surface and the greater degree surface respectively. σ1, σ2 are standard deviations of the smaller degree surface and the greater degree surface. F1 is the F distribution for one-degree difference between the tested and fitting surfaces.

For assessing performance of ANNs the criteria of root mean square error (RMSE) and coefficient of determination (R2) are sufficient. RMSE, expressed in equation (15), shows how far predictions fall from actual values. The smaller the RMSE value, the closer the fit to the data.

RMSE = 1n i=1n(Yio -  Yip)2                                                                                                     (15)

R2 score is the amount of the variation in the target variable which is predictable from the explanatory variables as mathematically shown in equation (16). It is used to check how well observed results are reproduced by the model. Best possible score is 1.0.

R2 = i=1nYio -  Yi0Yip -  Yipi=1nYio -  Yi02 .  i=1nYip -  Yip2 2                                                                         (16)

In equations (15) and (16), Yio, Yip, Yio, and Yip are observed and predicted values and their respective average values; m is the number of observations or examples. The two error metrics are hard-coded with Python codes in JupyterLab.

Coding the Surface Models

An adjustment program (called geoid_polynomial_surface_lagos.m) was written MathLab® by this author, which implements polynomial surfaces by outputting their adjusted parameters and stochastic properties. Another program (called geoid_surface_lagos,ipynb) was written by the same author in JupyterLab® to implements MLP model and GRNN model. GRNN was implemented using pyGRNN library developed by GKDD (Geosciences and Knowledge Discovery in Data) research group of the University of Lausanne, Switzerland (https://github.com/federhub/pyGRNN).

Four functions named training, testing, complete_prediction and evaluation were hard-coded in JupyterLab and called each time a surface model was going to be implemented. Training trains the ANN model, fitting it with the training data so it learns the relationships among the features in the data. Testing predicts new values of the target variable. The performance of the model when fed with the test dataset shows how it will perform when it encounters new data in the world (Taiwo et al., 2023). In complete_prediction, the entire dataset is employed in prediction and the output is stored in a csv file. The function evaluation assesses the model performance with RMSE and R2.

The coefficients of each polynomial surface model were computed in Microsoft Excel® worksheet from the observed dataset and stored in different csv file. For example, multiquadratic polynomial surface is stored in geoid_surface_multiquadratic.csv. Each of the files holds the data input for the MLP and GRNN models, and is called into the codes.

MLP carries along a number of hyper-parameters which are listed in Table 2.

Table 2: ANN hyper-parameters, their descriptions

| Parameters | Description |
| --- | --- |
| hidden_layer_sizes | Number of hidden layers |
| activation | Activation function |
| solver | Optimizer |
| learning_rate_init | Learning rate |
| max_iter | Maximum number of iterations |

GRNN is coded with cross validation GridSearchCV in order to search through various values of sigma (σ) and use the value that gives output with the smallest RMSE. The hyper-parameters are listed in Table 3.

Table 3: GRNN hyper parameters and their descriptions

| Parameters | Description |
| --- | --- |
| Kernel | Weighting function |
| sigma | Spread constant |

The two error metrics – RMSE and R2 – are hard-coded in a function called evaluation written with Python in JupyterLab®. Each time MLP or GRNN is modeled, it calls evaluation function, which outputs the evaluation metrics RMSE and R2.

RESULTS AND DISCUSSION

geoid_polynomial_surface_lagos.m code output adjusted parameters ai, RMSE and R2 for each surface model. The parameters so obtained were used to form the mathematical model.

geoid_surface_lagos,ipynb code output RMSE and R2 for both MLP and GRNN. It should be noted that ANN models do not output parameters. Hence, unlike polynomial models, there is no mathematical model to represent the MLP and GRNN model outputs. However, like other machine learning models, ANN are good at prediction. Thus, both MLP and GRNN produced predicted or interpolated values of N, the precision of which is measured by RMSE.

According to Wang (2016), global gravity models, such as EGM08, can be used to compute geoid heights to an accuracy of a few centimeters. Results of polynomial geoid surface and ANN geoid surface are compared to geoid heights computed using EGM08.

Accuracies of the Modeling

RMSE and R2 values, outputs of the program described above, are listed in Table 4 and Table 5. The R2 values show that all the polynomials well fitted the data. It can also be seen that quintic polynomial gives the highest accurate prediction with lowest RMSE. Thus, in this study, Lagos geoid surface shall be modeled with quintic five-degree polynomial. It should be noted in Table 5 that the four polynomials provided input into the ANN models successively. This was done so as to observe how increase in the number of features influence the model performance. The number of terms in the polynomials provided the number of features input into the models. It should be recalled from equations (2), (3), (4) and (5) that plane surface, multiquadratic, third degree and quintic polynomials have 3, 8, 9 and 21 terms respectively. Hence, each provided 3, 8, 9 and 21 features in the ANN models respectively. It is a known fact that machine learning models tend to perform better with increase in number of features (Taiwo et al., 2023). This fact is seen in Table 5 as GRNN increases in performance with decrease in RMSE value from 0.036 to 0.015.

Table 4: Accuracy measures of the polynomial models

| Polynomial | RMSE | R2 |
| --- | --- | --- |
| Plane surface | 0.054 | 0.98 |
| Multiquadratic | 0.015 | 0.99 |
| Third degree | 0.016 | 0.99 |
| Quantic | 0.010 | 0.99 |

Table 5: Accuracy measures of the ANN models

|  | Plane surface |  | Multiquadratic |  | Third degree |  | Quintic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RMSE | R2 | RMSE | R2 | RMSE | R2 | RMSE | R2 |
| MLP | 0.039 | 0.99 | 0.058 | 0.99 | 0.106 | 0.93 | 0.174 | 0.86 |
| GRNN | 0.036 | 0.99 | 0.026 | 0.99 | 0.020 | 99 | 0.015 | 0.99 |

Parameters of Quintic Polynomial

Equation (17) is the mathematical representation of the Lagos geoid surface modelled with quintic five-degree polynomial:

N= a00+ a10x+a01y + a11xy+ a20x2+a02y2+ a21x2y+ a12xy2+a30x3+a03y3 + a22x2y2+a31x3y+a13xy3+a40x4+a04y4+a32x3y2+a23x2y3+a41x4y+a14xy4+a50x5+a05y5										(17)

The system of observation equations was computed with least squares adjustment technique and produced parameters or coefficients, which are as listed in Table 6.

Table 6: Values of parameters of quintic polynomial for computing geoid heights

| Param | Value | Param | Value | Param | Value |
| --- | --- | --- | --- | --- | --- |
| a00 | -70.570e+003 | a12 | 399.303e-015 | a04 | 388.532e-021 |
| a10 | 56.604e-003 | a30 | -851.802e-021 | a32 | -264.376e-027 |
| a01 | 303.370e-003 | a03 | -64.812e-015 | a23 | 601.967e-027 |
| a11 | -9.5827e-009 | a22 | -108.416e-015 | a41 | -67.485e-027 |
| a20 | -357.114e-009 | a31 | 520.978e-021 | a14 | -279.911e-027 |
| a02 | -277.366e-009 | a13 | 102.121e-021 | a50 | 55.223e-027 |
| a21 | 180.458e-015 | a40 | -106.303e-021 | a05 | -153.045e-027 |

Enrichment of EGM08 for Lagos State Geoid

Enrichment of global geoid models such as EGM08 is necessary to remove systematic errors and local abnormalities inherent in the geoid heights values produced by the models. The values of the geoid heights N produced by EGM08 were adjusted with geoid height values which were determined (or observed) by Lagos State mapping. This is necessary to enrich the global model with terrestrial data. Thus, the global model will provide more accurate values of geoid heights in Lagos State. The following model was used for this adjustment:

∆NEGM=NOBS-NEGM = a00+ a10x+a01y + a11xy+ a20x2+a02y2+ a21x2y+ a12xy2+a30x3+a03y3 + a22x2y2+a31x3y+a13xy3+a40x4+a04y4+a32x3y2+a23x2y3+a41x4y+a14xy4+a50x5+a05y5						(18)

Note in equation (18) that the residuals between observed geoid heights and EGM08 geoid heights (that is, NOBS-NEGM) became the observations in  the observation equations. The system of observation equations was computed with least squares adjustment technique, which produced parameter values listed in Table 7. The computation resulted in a correction surface, ∆NEGM which represents the adjustment which must be applied to the EGM08 geoid heights in order to coincide with the corresponding geoid heights of Lagos State.

Table 7: Values of parameters of quintic polynomial for enriching EGM08

| Param | Value | Param | Value | Param | Value |
| --- | --- | --- | --- | --- | --- |
| a00 | -16.787e+003 | a12 | -654.485e-015 | a04 | -35.7753e-021 |
| a10 | -55.958e-003 | a30 | 154.520e-021 | a32 | -926.059e-027 |
| a01 | 80.297e-003 | a03 | 702.441e-015 | a23 | 656.858e-027 |
| a11 | -197.250e-009 | a22 | 184.560e-015 | a41 | 288.322e-027 |
| a20 | -225.777e-009 | a31 | 700.506e-021 | a14 | -447.879e-027 |
| a02 | 584.156e-009 | a13 | 537.530e-021 | a50 | 247.800e-027 |
| a21 | -722.594e-015 | a40 | -879.331e-021 | a05 | 27.873e-027 |

Table 8 and Table 9 show the results of enriching EGM08 in order to make it correspond to Lagos State geoid. The correction ∆NEGM computed from equation (18) was added to EGM08 geoid heights to achieve the enrichment. the difference in systematic error before and after the enrichment is clear from Table 9. The systematic error was reduced from an average of 0.13m to 0.01m. This error is due not only to bad fitting of the EGM2008 to the local geoid of Lagos State but also to the different zero geoid surfaces defined by the global model and the local datum which was used for determination of the orthometric heights (Alevizakolu and Lambrou, 2011).

Table 8: Sample result of enriching EGM08 in order to correspond to Lagos State geoid

| ID | OTHO- METRIC HEIGHT H | ELLIP- SOIDAL HEIGHT h | OBSD GEOID HEIGHT NOBS | COMP GEOID HEIGHT NCOMP | RES: OBSD- COMP ∆NOBS_COMP | EGM08 GEOID HEIGHT NEGM | RES: OBSD- EGM ∆NOBS_EGM | EGM08 CORR ∆NEGM | EGM08 ENRICHED NEGM+∆NEGM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XST101 | 3.608 | 26.063 | 22.455 | 22.457 | 0.0021 | 22.27 | 0.185 | 0.2224 | 22.4924 |
| XST102 | 3.424 | 26.089 | 22.666 | 22.671 | 0.0052 | 22.47 | 0.196 | 0.2149 | 22.6849 |
| XST103 | 18.757 | 41.872 | 23.115 | 23.104 | 0.0108 | 22.95 | 0.165 | 0.1707 | 23.1207 |
| XST104 | 32.178 | 55.482 | 23.303 | 23.291 | 0.0115 | 23.17 | 0.133 | 0.1493 | 23.3193 |
| XST105 | 28.526 | 52.053 | 23.527 | 23.517 | 0.0096 | 23.42 | 0.107 | 0.1293 | 23.5493 |
| XST106 | 4.518 | 27.387 | 22.868 | 22.867 | 1E-04 | 22.68 | 0.188 | 0.1925 | 22.8725 |
| XST107 | 3.34 | 25.47 | 22.129 | 22.132 | 0.0034 | 21.96 | 0.169 | 0.2091 | 22.1691 |
| XST109 | 8.168 | 30.897 | 22.728 | 22.732 | 0.0047 | 22.55 | 0.178 | 0.1927 | 22.7427 |
| XST110 | 26.23 | 49.217 | 22.987 | 23.000 | 0.013 | 22.84 | 0.147 | 0.1646 | 23.0046 |
| XST111 | 14.877 | 37.956 | 23.079 | 23.081 | 0.0028 | 22.92 | 0.159 | 0.1624 | 23.0824 |

Table 9: Systematic error in EGM2008 before and after enrichment

| ID | OBSD GEOID HEIGHTS | SYS-ERROR BEFORE ENRICHMENT | EGM2008 ENRICHED | SYS-ERROR AFTER ENRICHMENT |
| --- | --- | --- | --- | --- |
| XST101 | 22.455 | 0.2224 | 22.4924 | -0.0374 |
| XST102 | 22.666 | 0.2149 | 22.6849 | -0.0189 |
| XST103 | 23.115 | 0.1707 | 23.1207 | -0.0057 |
| XST104 | 23.303 | 0.1493 | 23.3193 | -0.0163 |
| XST105 | 23.527 | 0.1293 | 23.5493 | -0.0223 |
| XST106 | 22.868 | 0.1925 | 22.8725 | -0.0045 |
| XST107 | 22.129 | 0.2091 | 22.1691 | -0.0401 |
| XST109 | 22.728 | 0.1927 | 22.7427 | -0.0147 |
| XST110 | 22.987 | 0.1646 | 23.0046 | -0.0176 |
| XST111 | 23.079 | 0.1624 | 23.0824 | 0.0034 |

Figure 3 graphically shows the relationship between observed, computed and EGM08 geoid heights. (a) shows two facts: One, computed and observed geoid heights are minimally different as the two graphs (blue and pink) practically merged into one. Two, difference between observed and EGM08 geoid heights before enrichment is obvious as the EGM08 graph (grey) clearly shows. (b) shows how the three graphs almost merge into one after enriching EGM08.

(a)

(b)

Figure 3: Geoid Heights (a) before EGM08 enrichment and (b) after EGM08 enrichment

According to these results, a surveyor can determine orthometric height of any point in Lagos State without rigor. He will simply measure ellipsoidal heights with GPS, input the eastings and northings into equation (17), compute geoid height NCOMP, and simply subtract the computed geoid height from the observed ellipsoidal height as shown in equation 19:

H=h-NCOMP										(19)

The uncertainty of using this method is about ±1cm.

Additionally, a professional who downloads EGM08 geoid height, can enrich it by computing the correction ∆NEGM using equation (18) and adding the correction to the downloaded value, as in equation (20):

N=NEGM+∆NEGM										(20)

The uncertainty of using this method is also about ±1cm.

Comparing with the Accuracies Achieved in Existing Studies

The accuracy achieved with the method described in this study is better than the accuracies of similar studies reported in literature. Nwilo et al., (2009) got RMSE of 6cm. Odumosu et al., (2016) achieved RMSE of 9cm. In a study by Oluyori et al. (2019) in developing geoid model used for transforming ellipsoidal to orthometric heights via GPS levelling experimented with multiquadratic and bi-cubic polynomials. RMSE of the multiquadratic was ±11 cm and that of bi-cubic was of ±14 cm. Odumosu et al., (2016) developed empirical geoid model along with gravimetric geoid model for Lagos State. The obtained gravimetric geoid had a RMSE of 2.37cm when compared with GPS/Levelling geoid of the same area. Olaleye et al., (2013) got RMSE of 15cm using a plane surface polynomial.

CONCLUSION

This study models the local geoid surface in Lagos State Nigeria with 2-, 3-, 4- and 5-degree polynomials, and two ANN models: MLP and GRNN, using observed geoid heights sourced from the Office of the State Surveyor General and EGM08-derived geoid heights. The results were compared. EGM08-derived geoid heights were enriched with the observed geoid heights in order to make it more accurately represent the local geoid. Performance of these models in optimally representing the geoid in the study area was investigated. Quintic polynomial model produced the lowest RMSE of 1cm. When applied to GRNN, the ANN model performed with an RMSE of 1.5cm.

REFERENCE

Akar Alper et al.: 2020, Local geoid determination using a generalized regression neural network and interpolation methods: A case study in Kars, Turkey

Akcin, H. and Celik, C.T.: 2013, Performance of artificial neural networks on Kriging method in modeling local geoid. Bol. Cienc. Geod., 19, No. 1, 84–97. DOI: 10.1590/S1982-21702013000100006

Albayrak, M., Özlüdemir, M.T., Aref, M.M. and Halicioglu, K.: 2020, Determination of Istanbul geoid using GNSS/levelling and valley cross levelling data. Geod. Geodyn., 11, No. 3, 163–173. DOI: 10.1016/j.geog.2020.01.003

Alevizakolu, E and Lambrou, E.: 2011, Fast and Convenient Determination of Geoid Undulation (N) in Urban Area. FIG Working Week, Marrakech, Morroco,18-22 May 2011

Amos, M. & Featherstone, W.: 2009, Unification of New Zealand’s local vertical datums: iterative gravimetric quasigeoid computations. Journal of Geodesy. 83(1):57-68.

Awange, J. I., Grafarend, E. W., Palanez, B. and Zaletnyik, P.: 2010, Algebraic Geodesy and Geoinformatics. Springer and Business Media, 2010.

Ayeni, O. O. (2011). An Investigation into the Choice of the Most Appropriate Least Squares Method for Position Determination. In Contemporary Issues in Surveying and Geoinformatics. F. A. Fajemirokun (ed.) pp. 247 – 285, BPrint, Lagos.

Blitzkow, D., Matos, A., Guimarães, G. & Lobianco, M.: 2010, Recent progress of the geoid in South America. Presentado En La Reunión SIRGAS

Cakir Layla et al.: 2014, Polynomials, Radial Basis Function, and Multilayer Perceptron Neural Networks Methods in Local Geoid Determination with GPS/Levelling. ScienceDirect https://doi.org/10.1016/j.measurement.2014.08.003

Erol Bihter: 2011, Scientific Research and Essays Vol. 6(6), pp. 1351-1363, http://www.academicjournals.org/SRE  DOI: 10.5897/SRE10.1119 ISSN 1992-2248

Erol, B. and Erol, S.: 2013, Learning-based computing techniques in geoid modeling for precise height transformation. Comput. Geosci., 52, 95–107.

Eteje Sylvester Okiemute and Oduyebo Fatai Olujimi (2018) Local Geometric Geoid Models Parameters and Accuracy Determination Using Least Squares Technique INTERNATIONAL JOURNAL OF INNOVATIVE RESEARCH & DEVELOPMENT DOI No. : 10.24940/ijird/2018/v7/i7/JUL18098

Eteje, S. O., Oduyebo, O. F. and Oluyori, P. D.: 2019, Relationship between Polynomial Geometric Surfaces Terms and Observation Points Numbers and Effect in the Accuracy of Geometric Geoid Models. International Journal of Environment, Agriculture and Biotechnology (IJEAB), 4(4), 1181-1194. DOI: https://dx.doi.org/10.22161/ijeab.4444.

Featherstone W.: 1998, Do we need a gravimetric geoid or a model of the Australian height datum to transform GPS heights in Australia? Austr Surv 43(4): 273±280

Featherstone, W., Kirby, J., Kearsley, A., Gilliland, J., Johnston, G., Steed, J., Forsberg, R. & Sideris, M.: 2001, The AUSGeoid98 geoid model of Australia: data treatment, computations and comparisons with GPS-levelling data. Journal of Geodesy. 75(5- 6):313-330.

Fotopoulos Georgia: 2003, An Analysis on the Optimal Combination of Geoid, Orthometric and Ellipsoidal Height Data UCGE Reports Number 20185 Department of Geomatics Engineering, University of Calgary. URL: http://www.geomatics.ucalgary.ca/links/GradTheses.html

Ghilani, C. D. (2018). Adjustment Computations: Spatial Data Analysis. Sixth Edition. John Wiley

Gomaa D., Hoda F., Sherine I. (2010), Evaluation and adaptation of the EGM2008 Geopotential Model along the Northern Nile Valley, Egypt : Case study, Journal of Surveying Engineering

Haidar, A.M., Mustafa, M.W., Ibrahim, F.A. and Ahmed, I.A.: 2011, Transient stability evaluation of electrical power system using generalized regression neural networks. Appl. Soft Comput., 11, No. 4, 3558–3570. DOI: 10.1016/j.asoc.2011.01.028

Kirici, U. and Sisman, Y. (2017). The Comparison of the Adjustment Methods in Geoid Determination Method. FIG Working Week, Helsinki, Finland.

Kisi, O. (2008). The potential of different ANN techniques in evapotranspiration modelling. Hydrol. Process., 22, No. 14, 2449–2460. DOI: 10.1002/hyp.6837

Konakoglu B, Akar A: 2021, Geoid undulation prediction using Gaussian Processes Regression: A case study in a local region in Turkey. Acta Geodyn. Geomater., 18, No. 1 (201), 15–28, 2021. DOI: 10.13168/AGG.2021.0001

Kotsakis C., Sideris M. G.: 1999, On the adjustment of combined GPS/levelling/geoid networks Journal of Geodesy (1999) 73: 412±421

Kirici, U. and Sisiman, Y. (2017). The Comparison of the Adjustment Methods in Geoid Determination Method. FIG Working Week 2017.

Lambrou E., Pantazis G.: 2005, Local geoid modelling via the geometric interpolation method, 17th International Symposium of FIG Modern technologies, education and professional practice in Geodesy and related fields, Sofia Bulgaria, p.p. 40-47

Lin, C., Li, T., Chen, S., Liu, X., Lin, C. and Liang, S.: 2019, Gaussian process regression-based forecasting model of dam deformation. Neural. Comput. Appl., 31, No. 12, 8503–8518. DOI: 10.1007/s00521-019-04375-7

Lin, L.S.: 2007, Application of a back-propagation artificial neural network to regional grid-based geoid model generation using GPS and leveling data. J. Surv. Eng., 133, No. 2, 81–89. DOI: 10.1061/(ASCE)0733-9453(2007)133:2(81)

Mevlut Gullu, Mustafa Yilmaz and Ibrahim Yilmaz: 2011, Application of Back Propagation Artificial Neural Network for Modelling Local GPS/Levelling Geoid Undulations: A Comparative Study FIG Working Week 2011 Bridging the Gap between Cultures Marrakech, Morocco, 18-22 May 2011

Nwilo P. C.: 2009, GEOID MODELLING USING GEOMETRICAL INTERPOLATION METHOD. University of Lagos. https://www.researchgate.net/publication/266731752_GEOID_MODELLING_USING_GEOMETRICAL_INTERPOLATION_METHOD

Odumosu Joseph, Kelly Kevin, Adejare Quadri, Adeleke Oluwafemi and Olaniyi Abdulmalik: 2016, Empirical Geoid Modelling Using Classical Gravimetric Method. (8076) FIG Working Week 2016 Recovery from Disaster Christchurch, New Zealand, May 2–6, 2016

Okiwelu A. A., Okwueze E. E., Ude I. O.: 2011, Determination of Nigerian Geoid Undulations from Spherical Harmonic Analysis Applied Physics Research Vol. 3, No. 1; May 2011 8 ISSN 1916-9639 E-ISSN 1916-9647 www.ccsenet.org/apr

Olaleye, J. B., Badejo, O. T., Olusina J. O., and Aleem, K. F. 2013. Rectangular ‘Satlevel’ Model for Quick Determination of Geoid: A Case Study of Lagos State of Nigeria Journal of Emerging Trends in Engineering and Applied Sciences (JETEAS) 4(5):699-706 (ISSN: 2141-7016)

Oluyori, P. D. et al.: 2020, Improving The Local Geometric Geoid Model of FCT Abuja Accuracy by Fitting a Higher Order/Degree Polynomial Surface. FUDMA Journal of Sciences (FJS) Vol. 4 No. 3, September, 2020, pp 114 – 120 DOI: https://doi.org/10.33003/fjs-2020-0403-276

Roman, D.R., Wang, Y.M., Saleh, J. & Li, X.: 2009, National geoid height models for the United States: USGG2009 and GEOID09. Proceedings of the ACSM-MARLS-UCLSWFPS Conference, Salt Lake City, UT, USA. 20.

Sanlioglu, I., Maras, S. S. and Uysal, F.: 2009, Determination of Orthometric Heights with RealTime Kinematic Surveying, Konya Sample, FIG Working Week, 2009.

Specht, D.F.: 1991, A general regression neural network. IEEE Trans. Neural Netw., 2, No. 6, 568–576. DOI: 10.1109/72.97934

Wang Y. M. 2016. Geodetic Boundary Value Problems. Springer International Publishing Switzerland (Outside the USA) 2016 E.W. Grafarend (ed.), Encyclopedia of Geodesy, DOI 10.1007/978-3-319-02370-0_42-1

Yuan, Q., Li, S., Yue, L., Li, T., Shen, H. and Zhang, L.: 2019, Monitoring the variation of vegetation water content with machine learning methods: Point–surface fusion of MODIS products and GNSS-IR observations. Remote Sens., 11, No. 12, 1440. DOI: 10.3390/rs11121440

Zilkoski, D.B.: 1992, Special Report Results of the General Adjustment of the North American Vertical Datum of 1988 David B. Zilkoski, John H. Richards, and Gary M. Young. Surveying and Land Information Systems. 52(3):133-149.